Фрагменты исходного кода программы:

correlator_channel.v:

include(mpc_bus.v)

`timescale 1ns/10ps

`include "global_param.v"

module correlator_channel (

clk,

pclk,

reset_n,

s_reset,

we_en,

rd_en,

reg_addr,

wdata,

adc0_in,

adc1_in,

imi_in,

intr_pulse,

rdata,

epoch_pulse,

DBG

);

parameter BASE_ADDR = `ADDR_WIDTH'h8000;

input clk;

input pclk;

input reset_n;

input s_reset;

input we_en;

input rd_en;

input [`ADDR_WIDTH - 1 : 0] reg_addr;

input [31 : 0] wdata;

input [1 : 0] adc0_in;

input [1 : 0] adc1_in;

input [1 : 0] imi_in;

input intr_pulse;

output [31 : 0] rdata;

wor [31 : 0] rdata;     // Так надо для работы ../mpc_bus.v

output      epoch_pulse;

output [5 : 0] DBG;

wire [31 : 0] code_rate;

wire [31 : 0] code_phase_init;

wire [31 : 0] phase_rate;

wire doinit_pulse;

wire doinit_pulse_int;

wire [31 : 0] code_rate_int;

wire [31 : 0] code_phase_int;

wire [31 : 0] phase_rate_int;

wire [31 :0] phase_int;

wire [31 : 0] phase_cycles_int;

wire [4 : 0] cos_product;

wire [4 : 0] sin_product;

wire [4 : 0] phase_addr;

wire [4 : 0] promt_cos_product;

wire [4 : 0] promt_sin_product;

wire [4 : 0] early_cos_product;

wire [4 : 0] early_sin_product;

wire [4 : 0] earlyearly_cos_product;

wire [4 : 0] earlyearly_sin_product;

wire [4 : 0] late_cos_product;

wire [4 : 0] late_sin_product;

wire [4 : 0] latelate_cos_product;

wire [4 : 0] latelate_sin_product;

wire [15 : 0] promtq_int;

wire [15 : 0] promti_int;

wire [15 : 0] earlyq_int;

wire [15 : 0] earlyi_int;

wire [15 : 0] earlyearlyq_int;

wire [15 : 0] earlyearlyi_int;

wire [15 : 0] lateq_int;

wire [15 : 0] latei_int;

wire [15 : 0] latelateq_int;

wire [15 : 0] latelatei_int;

wire [5 : 0] delay1;

wire [5 : 0] delay2;

wire [1 : 0] iq_shift;

wire [1 : 0] input_reg;

wire [1 : 0] sample_int;

wire        epoch_pulse;

wire   [31:0]  channel_regfile_rdata;

assign rdata = channel_regfile_rdata;

channel_regfile

#(BASE_ADDR) CR (

.clk                  (clk),

.reset_n              (s_reset & reset_n),

.we_en                (we_en),

.rd_en                (rd_en),

.reg_addr             (reg_addr),

.wdata                (wdata),

.rdata                (channel_regfile_rdata),

.code_rate_int        (code_rate_int),

.code_phase_int       (code_phase_int),

.phase_rate_int       (phase_rate_int),

.phase_int            (phase_int),

.phase_cycles_int     (phase_cycles_int),

.promtq_int           (promtq_int),

.promti_int           (promti_int),

.earlyq_int           (earlyq_int),

.earlyi_int           (earlyi_int),

.earlyearlyq_int      (earlyearlyq_int),

.earlyearlyi_int      (earlyearlyi_int),

.lateq_int            (lateq_int),

.latei_int            (latei_int),

.latelateq_int        (latelateq_int),

.latelatei_int        (latelatei_int),

.input_reg            (input_reg),

.code_rate            (code_rate),

.code_phase_init      (code_phase_init),

.doinit               (doinit_pulse),

.phase_rate           (phase_rate),

.delay1               (delay1),

.delay2               (delay2),

.iq_shift             (iq_shift)

);

signal_mux CRMUX (

.input_reg    (input_reg),

.adc0_in      (adc0_in),

.adc1_in      (adc1_in),

.imi_in       (imi_in),

.signal_out   (sample_int)

);

channel_sync CHS (

.sclk         (clk),

.dclk         (pclk),

.reset_n      (s_reset & reset_n),

.doinit       (doinit_pulse),

.doinit_sync  (doinit_pulse_int)

);

channel_doinit CHDO (

.clk           (pclk),

.reset_n       (s_reset & reset_n),

.doinit_pulse  (doinit_pulse_int),

.intr_pulse    (intr_pulse),

.doinit        (doinit)

);

BUS_MODULE_DATA(SR)

channel_shift_reg

#(BASE_ADDR + 1) SR( // Базовый адрес генератора кода !!!

BUS_INST(SR),

.pclk                  (pclk),

.code_rate            (code_rate),

.code_phase_init      (code_phase_init),

.doinit               (doinit),

.intr_pulse           (intr_pulse),

.dly_epoch            (dly_epoch),

.code_out             (code_out),

.code_rate_int        (code_rate_int),

.code_phase_int       (code_phase_int),

.epoch_pulse          (epoch_pulse),

.DBG                  (DBG[4:0])

);

assign DBG[5] = phase_addr[4];

channel_synthesizer CHSYN (

.clk                  (pclk),

.reset_n              (s_reset & reset_n),

.phase_rate           (phase_rate),

.doinit               (doinit),

.intr_pulse           (intr_pulse),

.epoch_pulse          (dly_epoch),

.phase_rate_int       (phase_rate_int),

.phase_int            (phase_int),

.phase_cycles_int     (phase_cycles_int),

.phase_addr           (phase_addr)

);

channel_cos_table CHCT (

.adc           (sample_int),

.phase_addr    (phase_addr),

.cos_product   (cos_product)

);

channel_sin_table CHST (

.adc           (sample_int),

.phase_addr    (phase_addr),

.sin_product   (sin_product)

);

channel_delay_reg CHDR (

.clk               (pclk),

.reset_n           (s_reset & reset_n),

.code              (code_out),

.epoch_pulse       (epoch_pulse),

.delay1            (delay1),

.delay2            (delay2),

.promt_code        (promt_code),

.early_code        (early_code),

.earlyearly_code   (earlyearly_code),

.late_code         (late_code),

.latelate_code     (latelate_code),

.dly_epoch         (dly_epoch)

);

assign promt_cos_product = promt_code ? cos_product : (~cos_product + 1'b1);

assign promt_sin_product = promt_code ? sin_product : (~sin_product + 1'b1);

assign early_cos_product = early_code ? cos_product : (~cos_product + 1'b1);

assign early_sin_product = early_code ? sin_product : (~sin_product + 1'b1);

assign earlyearly_cos_product = earlyearly_code ? cos_product : (~cos_product + 1'b1);

assign earlyearly_sin_product = earlyearly_code ? sin_product : (~sin_product + 1'b1);

assign late_cos_product = late_code ? cos_product : (~cos_product + 1'b1);

assign late_sin_product = late_code ? sin_product : (~sin_product + 1'b1);

assign latelate_cos_product = latelate_code ? cos_product : (~cos_product + 1'b1);

assign latelate_sin_product = latelate_code ? sin_product : (~sin_product + 1'b1);

channel_adder CHAD (

.clk                       (pclk),

.reset_n                   (s_reset & reset_n),

.doinit                    (doinit),

.intr_pulse                (intr_pulse),

.epoch                     (dly_epoch),

.promt_cos_product         (promt_cos_product),

.promt_sin_product         (promt_sin_product),

.early_cos_product         (early_cos_product),

.early_sin_product         (early_sin_product),

.earlyearly_cos_product    (earlyearly_cos_product),

.earlyearly_sin_product    (earlyearly_sin_product),

.late_cos_product          (late_cos_product),

.late_sin_product          (late_sin_product),

.latelate_cos_product      (latelate_cos_product),

.latelate_sin_product      (latelate_sin_product),

.iq_shift                  (iq_shift),

.promtq_int                (promtq_int),

.promti_int                (promti_int),

.earlyq_int                (earlyq_int),

.earlyi_int                (earlyi_int),

.earlyearlyq_int           (earlyearlyq_int),

.earlyearlyi_int           (earlyearlyi_int),

.lateq_int                 (lateq_int),

.latei_int                 (latei_int),

.latelateq_int             (latelateq_int),

.latelatei_int             (latelatei_int)

);

endmodule

correlator_regfile.v:

`timescale 1ns/10ps

`include "correlator_param.v"

`include "global_param.v"

module correlator_regfile(

clk,

reset_n,

we_en,

rd_en,

reg_addr,

wdata,

rdata,

smpl_cntl,

smpl_cnth,

intr_cntl,

clksource,

intr_cntl_wr,

intr_per,

intr_dur,

prog_reset,

lamp

);

input clk;

input reset_n;

input we_en;

input rd_en;

input [`ADDR_WIDTH - 1 : 0] reg_addr;

input [31 : 0] wdata;

output [31 : 0] rdata;

input [31 : 0] smpl_cntl;

input [31 : 0] smpl_cnth;

output [3 : 0] intr_cntl;

output clksource;

output intr_cntl_wr;

output [`INTR_PERIOD_WIDTH - 1 : 0] intr_per;

output [`INTR_PULSE_DURATION_WIDTH - 1 : 0] intr_dur;

output prog_reset;

output lamp;

reg [31 : 0] rdata;

reg [3 : 0] intr_cntl;

reg intr_cntl_wr;

reg [`INTR_PERIOD_WIDTH - 1 : 0] intr_per;

reg [`INTR_PULSE_DURATION_WIDTH - 1 : 0] intr_dur;

reg prog_reset_int;

reg clksource;

reg lamp;

wire lamp_we;

wire intr_cntl_we;

wire intr_per_we;

wire intr_dur_we;

wire prog_reset_we;

wire prog_reset;

wire intr_cntl_rd;

wire intr_per_rd;

wire intr_dur_rd;

wire smpl_cntl_rd;

wire smpl_cnth_rd;

assign intr_cntl_we = (reg_addr == `INTR_CNTL_OFFSET) & we_en;

assign intr_cntl_rd = (reg_addr == `INTR_CNTL_OFFSET) & rd_en;

assign intr_per_we = (reg_addr == `INTR_PERIOD_OFFSET) & we_en;

assign intr_per_rd = (reg_addr == `INTR_PERIOD_OFFSET) & rd_en;

assign intr_dur_we = (reg_addr == `INTR_DURATION_OFFSET) & we_en;

assign intr_dur_rd = (reg_addr == `INTR_DURATION_OFFSET) & rd_en;

assign smpl_cntl_rd = (reg_addr == `SAMPLE_COUNTL_OFFSET) & rd_en;

assign smpl_cnth_rd = (reg_addr == `SAMPLE_COUNTH_OFFSET) & rd_en;

assign prog_reset_we = (reg_addr == `PROG_RESET_OFFSET) & we_en;

assign lamp_we = (reg_addr == `LAMP_OFFSET) & we_en;

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

lamp <= 1'b0;

end

else begin

if (lamp_we) begin

lamp <= wdata[0];

end

end

end

//Software reset register

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

prog_reset_int <= 1'b0;

end

else begin

if (prog_reset_we) begin

prog_reset_int <= wdata[0];

end

else begin

prog_reset_int <= 1'b0;

end

end

end

assign prog_reset = ~prog_reset_int;

//Process interrupt control register

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

intr_cntl <= 4'b0100;

end

else begin

if (intr_cntl_we) begin

intr_cntl <= wdata;

end

else begin

intr_cntl <= {1'b0,intr_cntl[2 : 0]};

end

end

end

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

clksource <= 1'b0;

end

else begin

if (intr_cntl_we) begin

clksource <= wdata[4];

end

end

end

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

intr_cntl_wr <= 1'b0;

end

else begin

intr_cntl_wr <= intr_cntl_we;

end

end

//Process interrupt period register

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

intr_per <= {`INTR_PERIOD_WIDTH{1'b0}};

end

else begin

if (intr_per_we) begin

intr_per <= wdata;

end

end

end

//Process interrupt duration register

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

intr_dur <= {`INTR_PULSE_DURATION_WIDTH{1'b0}};

end

else begin

if (intr_dur_we) begin

intr_dur <= wdata;

end

end

end

always @ (*) begin

rdata = {32{1'b0}};

case (1'b1)

intr_cntl_rd : rdata = {8'hab,19'b0,clksource,1'b0,intr_cntl[2 : 0]};

intr_per_rd  : rdata = intr_per;

intr_dur_rd  : rdata = intr_dur;

smpl_cntl_rd : rdata = smpl_cntl;

smpl_cnth_rd : rdata = smpl_cnth;

endcase

end

endmodule

channel_shift_reg.v:

`timescale 1ns/10ps

include(mpc_bus.v)

module channel_shift_reg (

BUS_PORTS,                  // Порты шины процессора

pclk,

code_rate,

code_phase_init,

doinit,

intr_pulse,

dly_epoch,

code_out,

code_rate_int,

code_phase_int,

epoch_pulse,

DBG

);

parameter BASE_ADDR = `ADDR_WIDTH'h8001;

// Сигналы доступа к шине процессора

BUS_PORTS_DESCR

input pclk;

input [31 : 0] code_rate;

input [31 : 0] code_phase_init;

input doinit;

input intr_pulse;

input dly_epoch;

output code_out;

output [31 : 0] code_rate_int;

output [31 : 0] code_phase_int;

output epoch_pulse;

output [4 : 0] DBG;

reg [31 : 0] code_phase;

reg [31 : 0] code_rate_int;

reg [31 : 0] code_phase_int;

wire epoch_pulse;

//Code frequency register

always @ (posedge pclk or negedge reset_n) begin

if (reset_n == 1'b0) begin

code_rate_int <= 32'b0;

end

else begin

if (((doinit == 1'b1) & (intr_pulse == 1'b1)) | epoch_pulse | (code_rate_int[31:0] == 32'h0)) begin

code_rate_int <= code_rate;

end

end

end

//Phase counter

always @ (posedge pclk or negedge reset_n) begin

if (reset_n == 1'b0) begin

code_phase <= 32'b0;

end

else begin

if ((doinit == 1'b1) & (intr_pulse == 1'b1)) begin

code_phase[31 : 0] <= code_phase_init;

end

else begin

code_phase[31 : 0] <= code_phase[31 : 0] + code_rate_int[31 : 0];

end

end

end

//Code phase regiter is latched when interrupt pulse asserted

always @ (posedge pclk or negedge reset_n) begin

if (reset_n == 1'b0) begin

code_phase_int <= 32'b0;

end

else if (intr_pulse == 1'b1) begin

code_phase_int[31 : 0] <= code_phase[31 : 0];

end

end

assign sr_shift = ((code_phase[31 : 0] + code_rate_int[31 : 0])) > 33'hFFFFFFFF;

// Модуль имитатора

BUS_MODULE_DATA(Imitator)

wire signed [`IMITATOR_BITS-1:0] imit_re;

wire signed [`IMITATOR_BITS-1:0] imit_im;

wire signed [11:0] 		    pure_out_re;

wire signed [11:0] 		    pure_out_im;

// Модуль генератора дальномерного кода

BUS_MODULE_DATA(CodeGenerator)

BOCgenerator

#(.CODE_GEN_BASE ( BASE_ADDR   ))  // Базовый адрес генератора кода !!!!!

CodeGenerator (

BUS_INST(CodeGenerator),               // Шина

.imi_clk              (pclk),

.doinit               (doinit),

.intr_pulse           (intr_pulse),       // <- Для начальной инициализации передаём doinit в прерывание

.dly_epoch            (dly_epoch),  // <- Эпоха возвращается обратно без задержки

.code_out             (code_out),

.epoch_pulse          (epoch_pulse),

.sr_shift             (sr_shift),       // <- Сигнал выработки нового чипа ПСП

.DBG                  (DBG)             // Отладочные сигналы

);

endmodule

channel_regfile.v:

`timescale 1ns/10ps

`include "global_param.v"

`include "channel_param.v"

module channel_regfile (

clk,

reset_n,

we_en,

rd_en,

reg_addr,

wdata,

rdata,

code_rate_int,

code_phase_int,

phase_rate_int,

phase_int,

phase_cycles_int,

promtq_int,

promti_int,

earlyq_int,

earlyi_int,

earlyearlyq_int,

earlyearlyi_int,

lateq_int,

latei_int,

latelateq_int,

latelatei_int,

input_reg,

code_rate,

code_phase_init,

doinit,

phase_rate,

delay1,

delay2,

iq_shift

);

parameter BASE_ADDR=`ADDR_WIDTH'h8000;

input clk;

input reset_n;

input we_en;

input rd_en;

input [`ADDR_WIDTH - 1 : 0] reg_addr;

input [31 : 0] wdata;

output [31 : 0] rdata;

input [31 : 0] code_rate_int;

input [31 : 0] code_phase_int;

input [31 : 0] phase_rate_int;

input [31 : 0] phase_int;

input [31 : 0] phase_cycles_int;

input [15 : 0] promtq_int;

input [15 : 0] promti_int;

input [15 : 0] earlyq_int;

input [15 : 0] earlyi_int;

input [15 : 0] earlyearlyq_int;

input [15 : 0] earlyearlyi_int;

input [15 : 0] lateq_int;

input [15 : 0] latei_int;

input [15 : 0] latelateq_int;

input [15 : 0] latelatei_int;

output [1 : 0] input_reg;

output [31 : 0] code_rate;

output [31 : 0] code_phase_init;

output doinit;

output [31 : 0] phase_rate;

output [5 : 0] delay1;

output [5 : 0] delay2;

output [1 : 0] iq_shift;

reg [31 : 0] code_rate;

reg [31 : 0] code_phase_init;

reg doinit;

reg [31 : 0] phase_rate;

reg [5 : 0] delay1;

reg [5 : 0] delay2;

reg [1 : 0] iq_shift;

reg [1 : 0] input_reg;

reg [31 : 0] rdata;

wire channel_config_we;

wire code_rate_we;

wire code_phase_init_we;

wire doinit_we;

wire phase_rate_we;

wire channel_config2_we;

wire channel_config_rd;

wire code_phase_init_rd;

wire code_rate_int_rd;

wire code_phase_int_rd;

wire phase_rate_int_rd;

wire phase_int_rd;

wire phase_cycles_int_rd;

wire ipqp_int_rd;

wire ie1qe1_int_rd;

wire il1ql1_int_rd;

wire ie2qe2_int_rd;

wire il2ql2_int_rd;

assign channel_config_we = (reg_addr == `CHANNEL_CONFIG_OFFSET) & we_en;

assign code_rate_we = (reg_addr == `CODE_RATE_OFFSET) & we_en;

assign code_phase_init_we = (reg_addr == `CODE_PHASE_INIT_OFFSET) & we_en;

assign doinit_we = (reg_addr == `CODE_DOINIT_OFFSET) & we_en;

assign phase_rate_we = (reg_addr == `PHASE_RATE_OFFSET) & we_en;

assign channel_config2_we = (reg_addr == `CHANNEL_CONFIG2_OFFSET) & we_en;

assign channel_config_rd = (reg_addr == `CHANNEL_CONFIG_OFFSET) & rd_en;

assign code_phase_init_rd = (reg_addr == `CODE_PHASE_INIT_OFFSET) & rd_en;

assign code_rate_int_rd = (reg_addr == `CODE_RATE_OFFSET) & rd_en;

assign code_phase_int_rd = (reg_addr == `CODE_PHASE_OFFSET) & rd_en;

assign phase_rate_int_rd = (reg_addr == `PHASE_RATE_OFFSET) & rd_en;

assign phase_int_rd = (reg_addr == `PHASE_OFFSET) & rd_en;

assign phase_cycles_int_rd = (reg_addr == `PHASE_CYCLES_OFFSET) & rd_en;

assign ipqp_int_rd = (reg_addr == `IPQP_OFFSET) & rd_en;

assign ie1qe1_int_rd = (reg_addr == `IE1QE1_OFFSET) & rd_en;

assign il1ql1_int_rd = (reg_addr == `IL1QL1_OFFSET) & rd_en;

assign ie2qe2_int_rd = (reg_addr == `IE2QE2_OFFSET) & rd_en;

assign il2ql2_int_rd = (reg_addr == `IL2QL2_OFFSET) & rd_en;

//Process registers

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

input_reg <= 2'b0;

end

else begin

if (channel_config_we) begin

input_reg <= wdata[9 : 8];

end

end

end

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

code_rate <= 32'b0;

end

else begin

if (code_rate_we) begin

code_rate <= wdata;

end

end

end

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

code_phase_init <= 32'b0;

end

else begin

if (code_phase_init_we) begin

code_phase_init <= wdata;

end

end

end

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

doinit <= 1'b0;

end

else begin

if (doinit_we) begin

doinit <= wdata[0];

end

else begin

doinit <= 1'b0;

end

end

end

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

phase_rate <= 32'b0;

end

else begin

if (phase_rate_we) begin

phase_rate <= wdata;

end

end

end

always @ (posedge clk or negedge reset_n) begin

if (reset_n == 1'b0) begin

delay1 <= 6'b0;

delay2 <= 6'b0;

iq_shift <= 2'b0;

end

else begin

if (channel_config2_we) begin

delay1 <= wdata[5 : 0];

delay2 <= wdata[13 : 8];

iq_shift <= wdata[26 : 24];

end

end

end

always @ (*) begin

rdata = {32{1'b0}};

case (1'b1)

channel_config_rd       : rdata = {8'hcc,8'b0,4'b0,{2'b0,input_reg},8'b0};

code_phase_init_rd      : rdata = code_phase_init;

code_rate_int_rd        : rdata = code_rate_int;

code_phase_int_rd       : rdata = code_phase_int;

phase_rate_int_rd       : rdata = phase_rate_int;

phase_int_rd            : rdata = phase_int;

phase_cycles_int_rd     : rdata = phase_cycles_int;

ipqp_int_rd             : rdata = {promtq_int,promti_int};

ie1qe1_int_rd           : rdata = {earlyq_int,earlyi_int};

il1ql1_int_rd           : rdata = {lateq_int,latei_int};

ie2qe2_int_rd           : rdata = {earlyearlyq_int,earlyearlyi_int};

il2ql2_int_rd           : rdata = {latelateq_int,latelatei_int};

endcase

end

endmodule